VITARKA  /  DEFENCE  /  POC-001  /  REV 2.0

ARC-Node

v2

A distributed passive edge-AI sensor network for counter-UAS alerting

— § —

Complete Technical Blueprint  ·  PoC v1 Build-Ready

● SYSTEM DEFINED · BUILD-READY        ▲ v1 PHASE ACTIVE

| PREPARED BY | Vitarka Labs |
| --- | --- |
| PRODUCT | ARC-Node v2 |
| DOCUMENT TYPE | Technical Blueprint / PoC Specification |
| VERSION | 2.0 — Frozen Architecture |
| STATUS | Build-Ready · v1 Phase Active |
| TARGET AUDIENCE | iDEX DISC · DRDO CAIR · Internal Engineering |
| DATE | 16 May 2026 |

Keywords: Counter-UAS · TinyML · GCC-PHAT · AES-CTR · LoRa · ESP32-S3 · INMP441 · AD8317 · Edge-AI · AoA

# § 01   Executive Summary

Synopsis

ARC-Node v2 is a phased, procurement-grade sensor network for passive counter-UAV alerting. It uses commodity ESP32-S3 microcontrollers with circular MEMS microphone arrays, passive RF energy detectors, and LoRa encrypted telemetry to identify and report drone threats from a perimeter deployment at a hardware cost of approximately ₹2,850 per node.

The architecture is organised into four hierarchical layers — Node, Compute, Network, Dashboard — and built on a staged deployment roadmap (v1 → v1.5 → v2 → v3) that avoids the engineering trap of building the full system at once. Each version is independently testable and producible.

Three pragmatic design corrections distinguish this blueprint from less-disciplined academic proposals: sector-based bearing estimation (not GPS-grade localisation), coarse RF energy corroboration (not RF protocol decoding), and AES-CTR + HMAC-SHA256 security (exploiting ESP32-S3 hardware accelerators correctly, without overclaiming GCM support).

A single ARC-Node costs ₹2,850 in components. A commercial radar-based C-UAS deployment costs ₹40L–4Cr per site. ARC-Node targets a 1,000× cost reduction for perimeter and border surveillance — while delivering two independent sensing modalities, on-device inference, encrypted telemetry, and a clear upgrade path to multi-node triangulation. — Strategic value proposition

# § 02   System Architecture

Four-Layer Hierarchical Design

The system is organised as four distinct layers. Each layer has well-defined inputs, outputs, and upgrade points. This separation enables modular testing — Layer A can be validated before Layer B is built.

┌─────────────────────────────────────────────────────────────────────┐ │ LAYER A · NODE 4× INMP441 · ESP32-S3 · AD8317 │ │ SX1276 LoRa │ ├─────────────────────────────────────────────────────────────────────┤ │ LAYER B · COMPUTE T0: Gatekeeper → T1: CNN → T2: AoA │ │ MFCC + 1D-CNN · GCC-PHAT │ ├─────────────────────────────────────────────────────────────────────┤ │ LAYER C · NETWORK AES-128-CTR · HMAC-SHA256 · Seq window │ │ 18-byte binary payload over LoRa │ ├─────────────────────────────────────────────────────────────────────┤ │ LAYER D · DASHBOARD LoRa GW → MQTT → Next.js + SOLR │ │ Confidence heatmap · health monitor │ └─────────────────────────────────────────────────────────────────────┘

### Layer A — Node Layer (Hardware Front-End)

| Component | Role | Notes |
| --- | --- | --- |
| 4 × INMP441 MEMS Mics | Circular acoustic array · I²S · 48 kHz / 24-bit | Precise geometry mount required for GCC-PHAT phase math |
| ESP32-S3 (Dual LX7) | Main compute · I²S DMA · Edge-AI · AES+HMAC crypto | Hardware AES (CTR), HMAC-SHA256, ULP co-processor |
| AD8317 RF Log Detector | Passive 1 MHz – 10 GHz wideband energy sensing | Corroboration gate only — not protocol identification |
| SX1276 LoRa Module | Encrypted binary telemetry · SPI · CRC | SF7–SF10 configurable, duty-cycle managed |

### Layer B — Compute Layer (Hierarchical Edge-AI Stack)

The compute layer uses three sequential tiers. Each tier only runs when the previous tier triggers, dramatically reducing average power consumption.

| Tier | Name | Function | Trigger Condition |
| --- | --- | --- | --- |
| T0 | Gatekeeper | Spectral baseline (SSTB). Low-cost anomaly detection. Always-on (ULP co-processor). | Always active |
| T1 | CNN Classifier | MFCC features + 1D-CNN. Distinguishes drone harmonics from biological noise. | On Tier 0 trigger |
| T2 | AoA Bearing | 4-mic GCC-PHAT cross-correlation. Angle-of-Arrival sector (± 10° resolution). | On Tier 1 confidence > threshold |

### Layer C — Network Layer (Secure Binary LoRa Protocol)

When the Compute Layer confirms a threat with sufficient confidence, the node constructs a fixed-length binary payload and transmits it via LoRa.

PAYLOAD STRUCTURE — 18 BYTES struct ArcPayload { uint8_t node_id; uint8_t threat_class; uint8_t confidence; int16_t azimuth_deg; uint32_t timestamp_unix; uint16_t sequence_id; uint8_t hmac_tag[8]; }

Security: AES-128-CTR encryption (hardware-accelerated on ESP32-S3) applied to the struct, followed by HMAC-SHA256 (hardware-accelerated) over the ciphertext — Encrypt-then-MAC. A sliding sequence window at the gateway rejects replay attacks. This correctly uses the ESP32-S3 hardware AES accelerator (CTR mode supported) without overclaiming GCM mode.

### Layer D — Dashboard Layer (Command & Control)

The LoRa gateway forwards decrypted packets via MQTT to a Next.js/React frontend backed by SOLR for high-speed event indexing. Dashboard features: live event timeline, per-node confidence heatmap, threat azimuth indicator, node health monitor, and export to PDF/CSV.

# § 03   Engineering Precision

Three Pragmatic Corrections

These corrections are not limitations — they are what make this blueprint defensible in front of a DRDO jury or an iDEX reviewer. Overselling leads to failed field trials. Precise claims survive scrutiny.

## Correction 1.  Localisation Scope

| ✗ Not this claim | ✓ This claim |
| --- | --- |
| Precise GPS-grade localisation of drone position | Bearing / Sector Estimation (AoA) — direction reported as azimuth ± 10° sector |

Technical basis: GCC-PHAT (Generalised Cross-Correlation with Phase Transform) on a 4-element circular microphone array yields direction-of-arrival, not absolute position. A 4-mic array with ~10 cm spacing at 150 m target range provides bearing resolution of approximately ± 5–15° — suitable for sector-level alerting. Full TDoA triangulation (absolute XY localisation) is a v3 roadmap item requiring multi-node deployment.

## Correction 2.  RF Sensing Scope

| ✗ Not this claim | ✓ This claim |
| --- | --- |
| Drone RF protocol identification (DJI OcuSync, FlySky, FrSky decode) | Coarse RF energy corroboration — 2.4 / 5.8 GHz burst activity gate |

Technical basis: AD8317 is a wideband logarithmic detector covering 1 MHz to 10 GHz with a 60 dB dynamic range. It outputs a voltage proportional to input power — it does not demodulate or decode protocols. The AD8317 serves as a cross-modal persistence gate: if the acoustic CNN fires AND the AD8317 reads elevated RF energy in the 2.4 / 5.8 GHz band, the fused confidence score rises. This reduces false positive rate without claiming RF fingerprinting.

## Correction 3.  Security Architecture

| ✗ Not this claim | ✓ This claim |
| --- | --- |
| AES-GCM (hardware-native authenticated encryption) | AES-CTR + HMAC-SHA256 (Encrypt-then-MAC) — equal security, correct implementation |

Technical basis: The ESP32-S3 Technical Reference Manual (v1.4) documents hardware AES acceleration for the following modes: ECB, CBC, OFB, CTR, CFB8, CFB128. AES-GCM is not listed in the hardware engine. AES-CTR is hardware-accelerated and, combined with HMAC-SHA256 (also hardware-accelerated), achieves IND-CPA + INT-CTXT security equivalent to GCM in the Encrypt-then-MAC construction. The sliding sequence window provides replay attack resistance equivalent to GCM's authentication nonce.

# § 04   Staged PoC Roadmap

Build sequence · v1 → v1.5 → v2 → v3

The staged build strategy is the single most important risk-control decision in this project. Each version is independently shippable and testable. Do not attempt to build v2 before v1 is validated.

| Version | Title | Build Scope | Success Gate |
| --- | --- | --- | --- |
| v1 ● | Foundation | ESP32-S3 data logger → Tier 0 gatekeeper → CNN classifier (TFLite Micro) → AES-CTR encrypted binary LoRa alert packet | Drone audio classified > 85% accuracy · alert packet received and decrypted at gateway |
| v1.5 | Direction | 4-mic circular mount fabrication → I²S DMA 4-channel capture → GCC-PHAT phase math → Bearing sector in LoRa payload | Bearing estimate within ± 15° for a moving sound source at 20–50 m · logged on dashboard |
| v2 | Corroboration | AD8317 integration → RF energy sampling on ADC → Cross-modal fusion gate in firmware → Fused confidence score | False positive rate drops below 5% in 1-hour ambient noise field trial |
| v3 | Multi-Node | 2–3 node mesh deployment → TDoA solver at gateway → Triangulated position estimate → Dashboard map overlay | Drone XY position estimated within 20 m of GPS ground truth at ≤ 100 m range |

## First Build — ESP32-S3 Data Logger Harness

The v1 build begins with the data logger — not the classifier. This is critical: synthetic augmentation only works if your noise floor is real. You need actual Indian ambient audio (Pune traffic, auto engines, construction, monsoon, cicadas) before training a model that works in the field.

- Hardware: ESP32-S3 dev board + 1 × INMP441 (I²S) + SPI SD card module

- Firmware: I²S DMA continuous capture → WAV file write to SD card (16 kHz, 16-bit, 10-minute chunks)

- Deployment: Roof or field deployment for 24–48 hours minimum

- Output: Labelled ambient corpus → feed into physics-based synthetic augmentation pipeline

FORMULA — SYNTHETIC AUGMENTATION Physics-based distance attenuation For each training sample: attenuate clean drone audio using Lp = Lw − 20·log₁₀(r) − α·r (where r = simulated distance in metres, α = air absorption coefficient for the frequency band), then mix with a randomly sampled ambient clip at the resulting SNR. Apply random pitch shift (± 2%), speed perturbation (± 5%), and channel reverb. This produces training samples that generalise to real deployment distances.

# § 05   Bill of Materials

PoC v1 Node — Low-SWaP Design

Low-SWaP (Size, Weight, and Power) design. All components domestically sourceable in India (RoboCraze, Robu.in, Electronicscomp, Amazon.in).

| Component | Function | Cost (₹) | Interface |
| --- | --- | --- | --- |
| ESP32-S3-WROOM-1 | Edge-AI compute · I²S DMA · HW crypto | 400 | I²S + SPI + UART |
| 4 × INMP441 MEMS | Omnidirectional acoustic array · 48 kHz | 600 | I²S (× 4, DMA) |
| SX1276 LoRa Module | Long-range encrypted binary telemetry | 500 | SPI |
| AD8317 Eval Board | Passive RF burst energy corroboration gate | 800 | ADC (analog) |
| 18650 Li-ion + TP4056 | Power management (PoC scale) | 300 | VBAT rail |
| Custom 3D-Printed Housing | 4-mic precision geometry mount (circular) | 250 | Mechanical |
| TOTAL PER NODE |  | ~ ₹2,850 |  |

Domestically sourceable, sub-₹3,000 per node, two independent sensing modalities, encrypted telemetry. This is a deployment-cost frontier, not a research curiosity. — Procurement rationale

# § 06   Validation Metrics

Research-Grade · Jury-Defensible

These metrics are the deliverable standard for PoC v1 field trial sign-off. Each is precisely scoped to the v1 capability. Do not claim v3 metrics for v1 hardware.

| Metric | Target | Test Condition | Measurement Method |
| --- | --- | --- | --- |
| FPR (False Positive Rate) | < 5% | 1-hour ambient field trial · no drone present | Count false alerts over 60-min recording · divide by total decision windows |
| Detection Recall | > 90% | 50 drone fly-overs at 50–150 m range | Ground truth: drone in air · ARC-Node alert within 1.5 s counts as TP |
| Bearing Accuracy (AoA) | ± 10° sector | Moving source at 20 m, 50 m, 100 m | Compare ARC-Node reported azimuth to GPS bearing of sound source |
| End-to-End Latency | < 1.5 s | Triggered audio playback at known timestamp | Δ: audio trigger timestamp → dashboard event display timestamp |
| Average Power Draw | < 500 mW | Continuous · Tier 0 active · no trigger | Current probe on VBAT rail · average over 30 minutes |

# § 07   Phase-Wise Implementation Checklist

## Phase 1 — Synthetic Dataset Pipeline (Foundation)

- Deploy ESP32-S3 + INMP441 data-logger to roof/field for 24 h minimum ambient capture

- Label captured clips: traffic, auto engine, wind, birds, rain, construction, human voice

- Write Python augmentation script: apply Lp = Lw − 20·log₁₀(r) − α·r distance attenuation

- Overlay clean drone audio clips (DroneSound / Kaggle) at computed SNR

- Apply augmentation: ± 2% pitch shift, ± 5% speed, random reverb, amplitude jitter

- Target: 4,000 clips (2,000 drone, 2,000 ambient); 80/20 train-test split

- Validate: confirm augmented clips sound plausible on headphones before training

## Phase 2 — One-Node CNN Classifier on ESP32-S3

- Train 1D-CNN on MFCC features (40 coefficients, 1 s window, 10 ms hop)

- Quantise to INT8 with TFLite post-training quantisation (model < 200 KB flash)

- Flash TFLite Micro to ESP32-S3 · validate Tier 0 and Tier 1 logic with recorded audio

- Measure: test accuracy (target > 91%), inference latency (< 120 ms), flash usage

- Stress test: play 1-hour ambient loop · count false triggers

## Phase 3 — GCC-PHAT Bearing Estimation

- Fabricate 4-mic circular mount with known inter-element spacing (target: 8–12 cm diameter)

- Implement 4-channel I²S DMA capture on ESP32-S3

- Implement GCC-PHAT cross-correlation for all mic pairs (6 pairs for 4 mics)

- Derive AoA bearing estimate from phase difference · output azimuth sector in degrees

- Validate with moving speaker at 20 m, 50 m, 100 m · compare to GPS bearing

- Integrate bearing into LoRa payload Azimuth_Angle field

## Phase 4 — Encrypted LoRa Packet + Dashboard

- Implement AES-128-CTR encryption using ESP32-S3 hardware AES (Espressif mbedTLS HAL)

- Implement HMAC-SHA256 using ESP32-S3 hardware SHA accelerator

- Implement sliding sequence window at gateway (reject seq_id replays within 60 s)

- Route decrypted packets from LoRa gateway to MQTT broker (Mosquitto)

- Build Next.js frontend: live event feed, azimuth display, node health, SOLR-backed timeline

- Measure end-to-end latency: target < 1.5 s from acoustic trigger to dashboard event

## Phase 5 — RF Corroboration Fusion + Multi-Node

- Integrate AD8317: read ADC voltage proportional to 2.4/5.8 GHz ambient energy level

- Implement cross-modal gate: require both acoustic confidence > 0.75 AND RF energy spike

- Measure false positive rate reduction vs acoustic-only: target FPR < 5%

- Deploy 2–3 nodes with overlapping coverage zones · test simultaneous alert handling

- Test network collision handling: randomised backoff for simultaneous TX

- Begin TDOA solver prototype at gateway (required for v3)

Each phase is independently shippable. Each phase has a measurable success gate. No phase begins until the prior one is signed off. This is the discipline that separates a deployable system from a research curiosity. — Engineering practice